DC-B1303

Architectural and Engineering Specifications

Version 1.0

(Feb. 11, 2016)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-B1303 is a Network Camera (IP Camera) designed and manufactured by IDIS. This camera provides 3 Megapixel (2304 x 1296) resolution at 30ips (images per second) with H.264/MJPEG compression. This camera is equipped with True Day/Night, PoE (IEEE 802.3af Class 2), Audio I/O, Alarm I/O and microSD/SDHC card backup.

General Specification

- The IP camera shall be equipped with 3 Megapixel 1/3" CMOS Sensor.

- The IP camera shall be equipped with an optional C/CS mount lens

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (Digital WDR) for improved video quality in high-contrast situations (69.5 dB).

- The IP camera shall utilize configurable 2DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 2 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have video out feature (NTSC/PAL).

- The IP camera shall have on board microSD/SDHC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2304 x 1296 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.264 and MJPEG** compression.
(** IDIS protocol only)

- The IP camera shall support quadruple streams in DirectIP protocol mode.

- The IP camera shall support triple streams in IDIS protocol mode.

- The IP camera shall conform to the ONVIF** Profile S Ver.2.4.0 standard. (** IDIS protocol only)

- The IP camera shall be equipped with embedded web server (IDIS Web**) which works independently using a Web Browser with ActiveX plug-in. (** IDIS Protocol only)

- The IP camera shall have IP filtering, HTTPS, SSL, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Motion Detection, Active Tampering Alarm and Trip Zone.

Protocol Specification: DirectIP and IDIS Protocol

- The IP camera shall have 2 protocol modes, DirectIP and IDIS Protocol, and DirectIP is set as main protocol by default.

- The protocol modes shall be selectable between DirectIP and IDIS protocol mode to meet specific needs with IDIS Discovery tool.

  - DirectIP Protocol

    - DirectIP protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

    - DirectIP protocol shall provide Quadruple streams.

    - The bitrate shall be automatically adjusted by recording profile of DirectIP NVR.

    - DirectIP protocol shall support H.264 only as primary compression.

  - IDIS Protocol

    - IDIS protocol shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

    - IDIS protocol shall provide triple streams.

    - IDIS protocol shall support H.264 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/3" CMOS

- Maximum Resolution: 2304 x 1296

- Scanning Mode: Progressive Scan

- Lens Type: Optional (C/CS mount)

- Iris Control: DC Auto Iris / Manual Iris

- Minimum Illumination:

  - Color: 0.4 lux @ F1.4

  - B/W: 0.01 lux @ F1.4

- S/N Ratio: 39 dB

- Maximum Frame Rate: 30ips @2304 x 1296

- Video Resolution:

  - DirectIP Protocol Mode: 2304x1296, 1920x1080, 1280x720, 704x480, 640x360, 352x240

  - IDIS protocol mode: 2304x1296, 1920x1080, 1280x720, 704x480, 352x240

- Video Compression : H.264, MJPEG** (** IDIS Protocol only)

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Multi-Video Streaming:

  - DirectIP Protocol Mode: Quadruple streams

  - IDIS Protocol Mode: Triple streams

- Dynamic Range: 69.5dB

- True Day & Night: Yes (ICR)

- Intelligent Video Analytic: Video Motion Detection, Audio Detection, Tampering, Trip Zone

- Analog Video Output: 1 RCA

Audio Specification

- Audio Compression Algorithm: G.726 (16kHz), G.711 u-Law (8kHz)

- Audio Input / Output: Line-in 1ea + Internal Mic 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols:

  - DirectIP Protocol Mode: DirectIP Protocol

  - IDIS Potocol Mode: RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

- Streaming Mode: Unicast, Multicast

Security Specification

- DirectIP Protocol Mode: SSL Encryption

- IDIS Protocol Mode: Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS, SSL Encryption

- Maximum User Access:

  - DirectIP protocol mode: Direct camera access is unavailable.

  - IDIS protocol mode: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, TripZone

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL, TLS

Environmental Specification

- Operating Temperature: -10°C ~ +50°C (+14°F ~ +122°F), *Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90% non-condensing

Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af class 2)

- Power Consumption: 4.8W

- Regulatory Approvals: FCC, CE(50130-4), KC

Mechanical Specification

- Dimensions (Ø x H): 75.6mm x 61mm x 129.3mm (2.98" x 2.4" x 5.09")

- Unit Weight: 0.4 kg (0.89 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Feb. 11, 2016 | Initial Release |