DC-D1223WX

Architectural and Engineering Specifications

Version 1.2

(Nov. 10, 2017)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

## Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

## General

### Product Description

DC-D1223WX is a Network Camera (IP Camera) designed and manufactured by IDIS. This camera provides Full HD (1920 x 1080) resolution at 30IPS (images per second) with H.264/MJPEG compression. This camera is equipped with Vari-focal lens, True Day/Night, PoE (IEEE 802.3af Class 2), True WDR, Audio I/O, Alarm I/O, microSD/SDHC card backup, Vandal-proof dome enclosure design and IP66 rated.

### General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor

- The IP camera shall be equipped with 3mm – 9mm Vari-focal lens, F1.2 – 2.3.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have Wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall utilize 2DNR and 3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be Vandal-Proof and IP rating 66 complied dome enclosure design.

- IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 2 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have built-in heater for continued use in subzero temperature conditions and utilize 12 VDC input to provide power.

- The IP camera shall have video out feature (NTSC/PAL).

- The IP camera shall have onboard microSD/SDHC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920 x 1080 at rates up to 30 images per second (IPS, WDR).

- The IP camera shall provide direct network connection using H.264 and MJPEG** compression (** IDIS Protocol only).

- The IP camera shall support Quadruple Streams in DirectIP protocol mode.

- The IP camera shall support Triple Streams in IDIS protocol mode.

- The IP camera shall conform to the ONVIF** Profile S Ver 2.4.0 standard (** IDIS Protocol only).

- The IP camera shall be equipped with embedded web server (IDIS Web**) which works independently using a Web Browser with ActivX plug-in (** IDIS Protocol only).

- The IP camera shall have IP filtering, HTTPS, SSL, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Motion Detection, Active Tampering alarm and Face Detection.

### Protocol Specification: DirectIP and IDIS Protocol

- The IP camera shall have 2 protocol modes, DirectIP and IDIS Protocol, and DirectIP is set as main protocol by default.

- The protocol modes shall be selectable between DirectIP and IDIS protocol mode to meet specific needs with IDIS Discovery tool.

  - DirectIP Protocol

    - DirectIP protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

    - DirectIP protocol shall provide Quadruple streams.

    - The bitrate shall be automatically adjusted by recording profile of DirectIP NVR.

    - DirectIP protocol shall support H.264 only as primary compression.

  - IDIS Protocol

    - IDIS protocol shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

    - IDIS protocol shall provide triple streams.

    - IDIS protocol shall support H.264 and MJPEG compression.

## Technical Specification

### Video Specification

- Image Sensor: 1/2.8" CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: Vari-focal (f= 3 ~ 9mm, F1.2 ~ 2.3)

- IRIS Control: DC Auto Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide : 111.0º(H), 57.8º(V), 133.8º(D)

  - Tele : 35.7º(H), 20.0º(V), 41.0º(D)

  - Pan/Tilt Range: Pan: -167° ~ 167°, Tilt: 0° ~ 70°, Rotate: -176° ~ 176°

- Minimum Illumination:

  - COLOR : 0.005 lux @ F1.2

  - B/W : 0.0002 lux @ F1.2

- S/N Ratio: 31.2dB(100lux, F2.8)

- Maximum Frame Rate: 60ips @ 1920 x 1080, 30fps : 1920 x 1080(WDR)

- Video Resolution:

  - DirectIP protocol mode: 1920x1080, 1280x720, 704x480, 640x360, 352x240

  - IDIS protocol mode: 1920x1080, 1280x720, 704x480, 352x240

- Video Compression : H.264, MJPEG** (** IDIS Protocol only)

- Video Compression Level: 4 levels - Basic, Standard, High, Very High

- Multi-Video Streaming:

  - DirectIP protocol mode: Quadruple streams

  - IDIS protocol mode: Triple streams

- Dynamic Range: 120 dB (True WDR)

- True Day & Night: Yes (ICR)

- Image Stabilizer: Yes (DIS)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Face Detection

- Analog Video Output: 1 Terminal Block

### Audio Specification

- Audio Compression Algorithm: G.726 (16kHz), G.711 u-Law (8kHz)

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

### Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols:

  - DirectIP Protocol Mode: DirectIP Protocol

  - IDIS Potocol Mode: RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, 802.1x (EAP)

- Streaming Mode: Unicast, Multicast

### Security Specification

- DirectIP Protocol Mode: SSL Encryption

- IDIS Protocol Mode: Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS, SSL Encryption

- Maximum User Access:

  - DirectIP protocol mode: Direct camera access is unavailable.

  - IDIS protocol mode: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

### Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Face Detection, Alarm in and Tampering

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL, TLS

## Environmental Specification

- Operating Temperature: -10°C ~ +50°C (+14°F ~ +122°F) / *Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90% non-condensing

- Vandal-proof Enclosure: yes

- Outdoor Ready: IP66

## Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af class 2)

- Power Consumption: 5.16W

- Regulatory Approvals: FCC, CE (50130-4), KC

## Mechanical Specification

- Dimensions (Ø x H): Ø160mm x 112mm (Ø6.3" x 4.41")

- Unit Weight: 0.87 kg (1.92 lb)

# Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.2 | Ray Sun | Nov. 10, 2017 | Add Image Stabilizer |
| 1.1 | Brandon Jo | Sep. 26, 2017 | Removed trip zone |
| 1.0 | Roy Lee | Feb. 13, 2016 | Initial Release |