DC-M4211WRX (2.8 / 3.3mm)

Architectural and Engineering Specifications

Version 1.1

(Feb. 10, 2025)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

## General

### Product Description

DC-M4211WRX is an Outdoor Dome type IP Camera designed and manufactured by IDIS. This camera provides Full HD (1920 x 1080) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with IR LED, Fixed-Focal lens, True Day/Night, PoE (IEEE 802.3af Class 3), microSD/SDHC/SDXC card backup (up to 256GB), Vandal-proof bullet enclosure design, IP67, IK10 rated and provides continuous operation in subzero temperature.

### General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 
- 2.8mm fixed-focal lens, F2.0
- 3.3mm fixed-focal lens, F2.0

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall have IR LED looming 10m.

- The IP camera shall support Fixed-Iris.

- The IP camera shall be Vandal-Proof and IP rating 67 and IK10 complied dome enclosure design

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3

- The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920 x 1080 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression.

- The IP camera shall support Triple streams

- The IP camera shall conform to the ONVIF.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS), and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm and Trip Zone.

### Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Triple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

## Technical Specification

### Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: 
- Fixed-focal (f=2.8, F2.0)
- Fixed-focal (f=3.3, F2.0)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - 2.8mm : 116.6º(H), 61.45º(V), 127.3º(D)

  - 3.3mm : 101.3º(H), 54.5º(V), 119.7º(D)

- Pan/Tilt Range: Pan: -5° ~ 5°, Tilt: 20° ~ 75°, Rotate: -90° ~ 90°

- Minimum Illumination:

  - COLOR : 0.1 lux @ F2.2

  - B/W: 0 lux (IR LED ON)

- S/N Ratio: more than 45 dB

- Maximum Frame Rate:

  - 30fps : 1920 x 1080(WDR)

- Video Resolution:

  - 16:9 : 1920 x 1080, 1280 x 720, 640 x 360, 352 x 240

  - 4:3 : 1440 x 1080, 960 x 720, 480 x 360, 272 x 240

- Video Compression : H.265, H.264, MJPEG.

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Multi-Video Streaming: Triple streams

- Dynamic Range: 120 dB

- True Day & Night: Yes (ICR)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm and Trip Zone

- Analog Video Output: 1 CVBS

### Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law, OPUS

- Audio Input / Output: 1(Internal MIC) / 0

### Network Specification

- Port: RJ-45, 10/100Base-T, 1 port

- Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

- Security: Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)  Remote Access Client: DirectIP NVR Connection, IDIS Web, IDIS Mobile, IDIS Solution Suite

- Recording Session Buffer (NLTSrec): Up to 60MB

- Edge Storage: micro SD/SDHC/SDXC, Smart Failover (Up to 256GB)

### Security Specification

- Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)

- Maximum User Access: 10 sessions for (Admin, Watch, etc.), 1 session for Recording, 1 session for Searching the recorded data in SD card

## Environmental Specification

- Operating Temperature: -30°C ~ +55°C (-22°F ~ +122°F)

- Operating Humidity: 0% to 90%

- Vandal-proof Enclosure: Yes, Outdoor Dome Camera

- Outdoor Ready: IK10 / IP67 supports

## Electrical Specification

- Power Source: PoE(IEEE 802.3af class 3)

- Power Consumption: 8.9W

- Regulatory Approvals: FCC, CE, MIL-STD-810G, EN50155(M12 model only)

## Mechanical Specification

- Dimensions (Ø x H x D): Ø104 x 49.44 x 122.17mm

- Unit Weight: 0.36kg(0.79lb)

# Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | Robin Park | May. 10, 2024 | Initial release |