DC-M4811X (3.07mm / 3.2mm)

Architectural and Engineering Specifications

Version 1.0

(Sep 30th, 2024)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

## General

### Product Description

DC-M4811X(3.07mm / 3.2mm) is an Indoor IP Camera designed and manufactured by IDIS. This camera provides 8MP(3840 x 2160) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with Fixed-Focal lens, True Day/Night, PoE (IEEE 802.3af Class 3), Vandal-proof bullet enclosure design and IP66 rated.

### General Specification

- The IP camera shall be equipped with 8 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 3.07mm, F1.8 / 3.2mm, F1.8 fixed-focal lens.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall support Fixed-Iris.

- The IP camera shall be Vandal-Proof and IP66 rating.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 3840 x 2160 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression.

- The IP camera shall support Quadruple streams

- The IP camera shall conform to the ONVIF Profile S Ver.2.4.0 standard.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS), and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm and Trip Zone.

### Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

## Technical Specification

### Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 3840 x 2160

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=3.07mm, F1.8 / 3.2mm, F1.8)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - f=3.07mm : 115º(H), 60º(V), 140º(D)

  - f=3.2mm : 104º(H), 58º(V), 121º(D)

- S/N Ratio: more than 45 dB

- Maximum Frame Rate:

  - 30fps : 3840 x 2160

  - 15fps : 3840 x 2160(WDR)

- Video Resolution:

  - 3840 x 2160, 1920 x 1080, 1280 x 720, 640 x 360

- Video Compression : H.265, H.264, MJPEG.

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streaming

- Dynamic Range: 120 dB

- True Day & Night: Yes (ICR)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm and Trip Zone

### Network Specification

- Port: RJ-45, 10/100Base-T, 1 port

- Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

- Security: Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)  Remote Access Client: DirectIP NVR Connection, IDIS Web, IDIS Mobile, IDIS Solution Suite

- Recording Session Buffer (NLTSrec): Up to 60MB

### Security Specification

- Signed firmware, Digest Authentication, Password Protection, User access log, Chained Fingerprint, IP filtering, HTTPS, SSL, IEEE 802.1X(EAP-TLS)

- Maximum User Access: 10 sessions for (Admin, Watch, etc.), 1 session for Recording, 1 session for Searching the recorded data in SD card

## Environmental Specification

- Operating Temperature: -10°C ~ +50°C (14°F ~ +122°F)

- Operating Humidity: 0% to 90%

- Vandal-proof Enclosure: Yes

- Outdoor Ready: IP66

- Shock & Vibration Resistant: MIL-STD-810H

## Electrical Specification

- Power Source: PoE(IEEE 802.3af class 3)

- Power Consumption: 7.9W

- Regulatory Approvals: FCC Part 15, Subpart B and ICES-003

## Mechanical Specification

- Dimensions (L x W x H):

  - DC-M4811X Extendable Bracket

    - Not Extended: 214.9mm x 134.6mm x 245.1mm (8.46” x 5.30” x 9.65”)

    - Extended: 214.9mm x 134.6mm x 308.6mm  (8.46” x 5.30” x 12.15”)

  - DC-M4811X Short Bracket : 214.9mm x 134.6mm x 120.4mm  (8.46” x 5.30” x 4.74”)

- Unit Weight:

  - DC-M4811X Extendable Bracket : 1.27 kg (2.8 lb)

  - DC-M4811X Short Bracket: 1 kg (2.2 lb)

# Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | Robin Park | Sep. 30, 2024 | Initial release |